Benchmark study of slamming loads in head seas for various ship hulls

Contacts: 	Shan Wang – shan.wang@centec.tecnico.ulisboa.pt and

Carlos Guedes Soares c.guedes.soares@centec.tecnico.ulisboa.pt

# Background

Quantification of uncertainties in the computation of ship motions and loads is a necessary prerequisite for the efficient application of structural reliability methods and for establishing design wave loads. The slamming induced whipping response is usually investigated together with ship global motions and wave loads by hydroelastic analysis. The differences between the slamming models usually affect the analysis of ship responses in waves.

Our previous benchmark study (Parunov et al., 2024) on the prediction of whipping responses of a warship model indicated that the uncertainty of rigid body responses was similar to previous benchmark studies without important discrepancies among different seakeeping code, however the calculated whipping responses in regular waves were considerably different. One of the challenges was the big differences on the predictions of bow and stern slamming loads across the methods. Huang et al. (2022) also found that the uncertainty in 2-node whipping loads was stronger than the 1st and 2nd harmonic responses using the CFD–FEA coupling method for the S175 model. Meanwhile, the numerical uncertainty due to discretization in the prediction of slamming pressures was higher than the structural deflection, as shown in Wang et al. (2021). These investigations highlighted the necessities of understanding the uncertainties in slamming loads prediction.

The present benchmark study aims at uncertainty quantification for slamming loads at the stern and bow by using different ship types with relatively recent hull designs. Three different hulls corresponding to a cruise ship, an LNG carrier and a chemical tanker, from EU Extreme Seas project will be utilized. Experimental analysis for ship motions and slamming loads for these ships can be found in various publications (Wang and Guedes Soares, 2016a, b; Wang et al. 2016; Ley and El Moctar, 2021; Wang and Guedes Soares, 2022; Klein et al. 2023)

# Experimental Data

Ship data and the tested regular wave conditions will be provided for the benchmark study. To minimize uncertainty in geometry representation, the ships’ wetted-surface 3D models will be supplied in IGS format. All additional information required for seakeeping analysis, including mass distribution, hydrostatic particulars, and the locations of pressure transducers, will also be fully documented.

In this study, only experiments conducted in regular head waves are considered. The measured responses include transfer functions for heave and pitch motions, as well as midship vertical wave bending moments. Slamming loads are recorded at selected locations near the bow and stern, and, for certain cases, green water loads on deck are also measured.

The LNG carrier and the chemical carrier are tested at zero speed. The cruise ship is tested at a full-scale speed of 6.0 kn. The test matrix and vessel geometries will be distributed to participants in stages.

# Expected results

Participants are invited to apply a range of numerical methods, including potential flow–based approaches, Computational Fluid Dynamics (CFD), Smoothed Particle Hydrodynamics (SPH), Moving Particle Methods (MPS/MAM), Arbitrary Lagrangian–Eulerian (ALE) formulations, as well as other hybrid techniques. The required outputs include heave and pitch motions, together with slamming pressures at specified locations, for ships advancing in regular head waves. The main particulars of the hull, test conditions and locations of the pressure transducers will be provided to the participants.

Only results obtained with adequately verified numerical settings should be submitted. It should be noted that the benchmark analysis will focus exclusively on model uncertainty, assuming that numerical discretization uncertainties have been properly minimized by each participant.

# Participants are requested to submit the following

- A Word document describing the numerical method used and the corresponding numerical setup (check Report_template)

- For each ship, time-series data (in model scale) for heave and pitch motions, as well as pressure measurements for each test case. Each file must include these columns: (Time, Heave, Pitch, Pressure 1, Pressure 2, …, Pressure 36). A minimum duration of 20 seconds must be included for each case. (check Results template_Case 11_IST.txt)

# Goal and Timeframe

The proposed benchmark study will be the first uncertainty study on slamming loads for several ships of different types. It will provide a rational basis for defining the uncertainty model of nonlinear wave loads for ship structural reliability studies. The results analysis and discussion will be combined with the previous benchmark study on “Benchmark study of linear wave induced motions and loads in head seas for various ship hulls” since the same ships were utilized. The time frame to have this Benchmark completed is the end of March 2027, so numerical results are expected to be ready by the end of 2026.

# Description of the model tests for LNGC

This section describes the ship model data for the LNG carrier, the test matrix (11 cases) in regular head waves, and the locations of the pressure transducers. The model scale is 1:70. All the values provided here are on a model scale.

Figure 1.  Coordinate system of the ship model.

Figure 2. Graphic illustration of the pressure transducers in the model test.

Table 1. Main particulars of the LNGC (Model scale = 1/70).

Table 2. Mass distribution of the ship model.

Table 3. Test matrix (Regular head waves) (β=180°) V=0 kn.

Table 4. Locations of pressure transducers.

Additional information could be found in the reference paper Klein et al. (2023).

Figure 1.  Coordinate system of the ship model.

Table 1 Main particulars of the LNGC (Model scale = 1/70)

| Parameter | Variable | Full scale | Model scale | Unit |
| --- | --- | --- | --- | --- |
| Displacement | Δ | 35615 | 0.104 | [t] |
| Length btw pp | LPP | 186.9 | 2.67 | [m] |
| Breadth | B0 | 30.38 | 0.434 | [m] |
| Draught | T(mean) | 8.4 | 0.12 | [m] |
| Block coeff. | Cb | 0.73 | 0.73 |  |
| Length over all (LoA) | Loa | 197.13 | 2.816 | [m] |

Table 2. Mass distribution of the ship model

|  | LNG tanker (Model scale) |  |  |
| --- | --- | --- | --- |
| Description | Segment 1 | Segment 2 | Total |
|  | (Bow) | (Stern) |  |
| Mass (t) | 48.777 | 55.054 | 103.831 |
| Xcg (m) | 1.826 | 0.938 | 1.355 |
| Ycg (m) | 0.000 | 0.000 | 0.000 |
| Zcg (m) | 0.118 | 0.117 | 0.118 |
| Ixx (kg.m²) | 1.248 | 1.432 | 2.681 |
| Iyy (kg.m²) | 6.745 | 7.816 | 34.811 |
| Izz (kg.m²) | 7.021 | 6.910 | 34.182 |

Table 3. Test matrix (Regular head waves) (β=180°) V=0 kn (Stokes III)

|  | λ/L | omega | omega_model | wave height | wave height_model |
| --- | --- | --- | --- | --- | --- |
| Case No. |  | ω (rad/s) | ω (rad/s) | H (m) | H (m) |
| Case 1 | 0.6 | 0.7414 | 6.2029 | 8 | 0.11429 |
| Case 2 | 0.8 | 0.6421 | 5.3718 | 11 | 0.15714 |
| Case 3 | 0.9 | 0.6053 | 5.0646 | 12 | 0.17143 |
| Case 4 | 1 | 0.5743 | 4.8047 | 13 | 0.18571 |
| Case 5 | 1.1 | 0.5475 | 4.5811 | 14 | 0.20000 |
| Case 6 | 1.2 | 0.5242 | 4.3861 | 16 | 0.22857 |
| Case 7 | 1.4 | 0.4854 | 4.0607 | 18 | 0.25714 |
| Case 8 | 1.6 | 0.4540 | 3.7985 | 20 | 0.28571 |
| Case 9 | 1.8 | 0.4280 | 3.5812 | 22 | 0.31429 |
| Case 10 | 2 | 0.4061 | 3.3975 | 24 | 0.34286 |
| Case 11 | 2.2 | 0.3872 | 3.2393 | 25 | 0.35714 |

Figure 2. Graphic illustration of the pressure transducers in the model test.

Table 4. Locations of the pressure transducers

| No. | X (m) | Z (m) | Position |  |
| --- | --- | --- | --- | --- |
| 1 | 2,730 | 0,220 | C1 | R1 |
| 2 | 2,700 | 0,220 | C2 | R1 |
| 3 | 2,700 | 0,195 | C2 | R2 |
| 4 | 2,670 | 0,220 | C3 | R1 |
| 5 | 2,670 | 0,195 | C3 | R2 |
| 6 | 2,670 | 0,170 | C3 | R3 |
| 7 | 2,640 | 0,220 | C4 | R1 |
| 8 | 2,640 | 0,195 | C4 | R2 |
| 9 | 2,640 | 0,170 | C4 | R3 |
| 10 | 2,640 | 0,145 | C4 | R4 |
| 32 | 0,000 | 0,140 | stern 3 | middle |
| 34 | 0,030 | 0,140 | stern 2 | middle |
| 35 | 0,060 | 0,140 | stern 1 | middle |
| 36 | 2,580 | 0,000 | C6 | keel |

# Report template

## 1. Participant Information

Researcher(s):

Institution:

## 2. Introduction

This section should provide an overview of the numerical approach adopted for the prediction of slamming loads and associated ship responses. The participant should briefly describe the background of the method and its relevance to the simulation of nonlinear wave-induced loads, particularly slamming phenomena.

Reference to previous applications of the method in similar contexts (e.g., ship motions, hydroelasticity, slamming, or wave loads) should be included, together with key literature supporting the approach. These references will be considered in the literature review of the benchmark study.

## 3. Numerical Method and setup

This section should describe briefly the numerical methodology and simulation setup, including the following points

The numerical approach should be introduced, specifying the type of method used (e.g., potential flow, CFD, SPH, MPS, ALE, or hybrid methods), together with its main assumptions and limitations, particularly in relation to slamming load prediction. The version of the solver.

Key aspects of the numerical implementation should be outlined, including time integration, spatial discretization, turbulence modelling (if applicable), and the treatment of the free surface.

The discretization strategy should be described in terms of mesh or particle type, resolution, and any sensitivity considerations. The computational domain and boundary conditions, including wave generation and domain size, should also be defined.

Computational resources (hardware and typical run time), as well as the time step and total simulation duration, should be provided. The simulation time must be sufficient to obtain statistically meaningful results, with a minimum duration of 20 seconds as required by the benchmark

# References

Abdelwahab, H. S.; Wang, S.; Parunov, J., and Guedes Soares, C. 2023. A New Model Uncertainty Measure of Wave-Induced Motions and Loads on a Container Ship with Forward Speed. Journal of Marine Science and Engineering, 11, 1042.

Huang, S., Jiao, J. and Guedes Soares, C., 2022. Uncertainty analyses on the CFD–FEA co-simulations of ship wave loads and whipping responses. Marine Structures, 82, p.103129.

Klein, M.; Wang, S.; Clauss, G. F., and Guedes Soares, C. 2023. Experimental study on the effect of extreme waves on a LNG carrier. Journal of Marine Science and Application. 22:52-74.

Ley J, el Moctar O.  2021 A Comparative Study of Computational Methods for Wave-Induced Motions and Loads. Journal of Marine Science and Engineering, 9(1):83.

Parunov, J., Guedes Soares, C., Hirdaris, S., Iijima, K., Wang, X., Brizzolara, S., Qiu, W., Mikulić, A., Wang, S. & Abdelwahab, H. 2022. Benchmark study of global linear wave loads on a container ship with forward speed. Marine Structures, 84, 103162, 20 doi:10.1016/j.marstruc.2022.103162.

Parunov, J., Badalotti, T., Feng, Q., Gu, X., Iijima, K., Ma, N., ... & Guedes Soares, C. 2024. Benchmark on the prediction of whipping response of a warship model in regular waves. Marine structures, 94, 103549.

Wang, S. and Guedes Soares, C. 2016a. Experimental and numerical study of the slamming load on the bow of a chemical tanker in irregular waves. Ocean Engineering. 111:369-383.

Wang, S. and Guedes Soares, C. 2016b. Stern slamming of a chemical tanker in irregular head waves. Ocean Engineering. 122:322-332.

Wang, S., Zhang, HD., and Guedes Soares, C. 2016. Slamming occurrence for a chemical tanker advancing in extreme waves modelled with the nonlinear Schrodinger equation. Ocean Engineering. 119:135-142.

Wang, S., Islam, H., and Guedes Soares, C. 2021. Uncertainty due to discretization on the ALE algorithm for predicting water slamming loads. Marine Structures. 80:103086.

Wang, S. and Guedes Soares, C. 2022. Analysis of the Experimental Data of Slamming Loads on an LNG Carrier in Abnormal Waves. 41st International Conference on Ocean, Offshore and Arctic Engineering (OMAE2022); Hamburg, Germany.  Paper No: OMAE2022-79037