This file describes the ship model data for the LNG carrier, the test matrix (11 cases) in regular head waves, and the locations of the pressure transducers. The model scale is 1:70. All the values provided here are on a model scale.

Figure 1.  Coordinate system of the ship model.

Figure 2. Graphic illustration of the pressure transducers in the model test.

Table 1. Main particulars of the LNGC (Model scale = 1/70).

Table 2. Mass distribution of the ship model.

Table 3. Test matrix (Regular head waves) (β=180°) V=0 kn.

Table 4. Locations of pressure transducers.

Additional information could be found in the reference paper:

Klein, Marco, Shan Wang, Günther Clauss, and Carlos Guedes Soares. "Experimental study on the effect of extreme waves on a LNG carrier." Journal of marine science and application 22, no. 1 (2023): 52-74.

Figure 1.  Coordinate system of the ship model.

Table 1 Main particulars of the LNGC (Model scale = 1/70)

| Parameter | Variable | Full scale | Model scale | Unit |
| --- | --- | --- | --- | --- |
| Displacement | Δ | 35615 | 0.104 | [t] |
| Length btw pp | LPP | 186.9 | 2.67 | [m] |
| Breadth | B0 | 30.38 | 0.434 | [m] |
| Draught | T(mean) | 8.4 | 0.12 | [m] |
| Block coeff. | Cb | 0.73 | 0.73 |  |
| Length over all (LoA) | Loa | 197.13 | 2.816 | [m] |

Table 2. Mass distribution of the ship model

|  | LNG tanker (Model scale) |  |  |
| --- | --- | --- | --- |
| Description | Segment 1 | Segment 2 | Total |
|  | (Bow) | (Stern) |  |
| Mass (t) | 48.777 | 55.054 | 103.831 |
| Xcg (m) | 1.826 | 0.938 | 1.355 |
| Ycg (m) | 0.000 | 0.000 | 0.000 |
| Zcg (m) | 0.118 | 0.117 | 0.118 |
| Ixx (kg.m²) | 1.248 | 1.432 | 2.681 |
| Iyy (kg.m²) | 6.745 | 7.816 | 34.811 |
| Izz (kg.m²) | 7.021 | 6.910 | 34.182 |

Table 3. Test matrix (Regular head waves) (β=180°) V=0 kn (Stokes III)

|  | λ/L | omega | omega_model | wave height | wave height_model |
| --- | --- | --- | --- | --- | --- |
| Case No. |  | ω (rad/s) | ω (rad/s) | H (m) | H (m) |
| Case 1 | 0.6 | 0.7414 | 6.2029 | 8 | 0.11429 |
| Case 2 | 0.8 | 0.6421 | 5.3718 | 11 | 0.15714 |
| Case 3 | 0.9 | 0.6053 | 5.0646 | 12 | 0.17143 |
| Case 4 | 1 | 0.5743 | 4.8047 | 13 | 0.18571 |
| Case 5 | 1.1 | 0.5475 | 4.5811 | 14 | 0.20000 |
| Case 6 | 1.2 | 0.5242 | 4.3861 | 16 | 0.22857 |
| Case 7 | 1.4 | 0.4854 | 4.0607 | 18 | 0.25714 |
| Case 8 | 1.6 | 0.4540 | 3.7985 | 20 | 0.28571 |
| Case 9 | 1.8 | 0.4280 | 3.5812 | 22 | 0.31429 |
| Case 10 | 2 | 0.4061 | 3.3975 | 24 | 0.34286 |
| Case 11 | 2.2 | 0.3872 | 3.2393 | 25 | 0.35714 |

Figure 2. Graphic illustration of the pressure transducers in the model test.

Table 4. Locations of the pressure transducers

| No. | X (m) | Z (m) | Position |  |
| --- | --- | --- | --- | --- |
| 1 | 2,730 | 0,220 | C1 | R1 |
| 2 | 2,700 | 0,220 | C2 | R1 |
| 3 | 2,700 | 0,195 | C2 | R2 |
| 4 | 2,670 | 0,220 | C3 | R1 |
| 5 | 2,670 | 0,195 | C3 | R2 |
| 6 | 2,670 | 0,170 | C3 | R3 |
| 7 | 2,640 | 0,220 | C4 | R1 |
| 8 | 2,640 | 0,195 | C4 | R2 |
| 9 | 2,640 | 0,170 | C4 | R3 |
| 10 | 2,640 | 0,145 | C4 | R4 |
| 32 | 0,000 | 0,140 | stern 3 | middle |
| 34 | 0,030 | 0,140 | stern 2 | middle |
| 35 | 0,060 | 0,140 | stern 1 | middle |
| 36 | 2,580 | 0,000 | C6 | keel |